Operator Instructions for Project GBT-26B-012 (AGBT/26B-087) — Salt in Sgr B2

- Two paths, one for Ku-band & one for K-band/KFPA

- Run map_K_nacl_withCal.py or map_Ku_NaCl_withCal.py

  - configuration, autopeakfocus, balance

  - map RA

  - autopeakfocus

  - map Dec

  - repeat

- When time is up, abort the script.

Starting early:

- As long as the source is up (above ~5 deg; peak elevation is ~24deg), you can start early

Finishing early:

  - Each OTF row scan takes under two minutes (48.5 s for Ku, 97 s for K).  Any full scan row is useful

Failures:

- AutoPeakFocus:

  - If no source is found, allow the routine to search a wider radius until one is found (this is only needed if you are allowing astrid to choose the source)

  - If AutoPeakFocus fails, abort the script and re-run it. If it fails 2 more times, call the on-call scientist.

- Balance:

  - If the balance fails, re-run the balance command up to 3 times

  - After 3 failures, call the on-call scientist

- Weather (or other) shutdown:

  - Re-run the script from the start if the session is interrupted; maps already completed still count toward the total, so nothing observed is wasted.

Project Contact(s) Info:

- PI: Adam Ginsburg

- Email 1: adam.g.ginsburg@gmail.com

- Email 2: adamginsburg@ufl.edu

- Phone 1 (home / work / cell): 303-667-3805

- Co-Is: Brett McGuire, Anthony Remijan